Physics 120 Lab 8 (2019) Bipolar Junction Transistors

The bipolar junction transistor (BJT), conceived by William Shockley and first realized by Morry Tanenbaum (1955), predated the realization (although not the conception) of the FET and is one of the most profound technological achievements of the age of electronics. Let us study some basic BJT circuits. Compared with the FET, which functions as a voltage-controlled current source, the BJT operates as a current amplifier or a current-controlled current source.

8-1 Transistor junctions are diodes

Here is a method for spot-checking a suspected bad transistor: the transistor must look like a pair of diodes when you test each junction separately. Of course, the BJT does not behave like two back-to-back diodes when operating as the base is physically very thin and draws little current.

Figure 8.1: Transistor junctions (for testing, not to describe transistor operation).

Get a 2N3904 NPN transistor, identify its leads, and verify that it looks like the diagram of back-to-back diodes in Fugure 8.1. Use a DMM’s diode test function (Note: indicated by a diode symbol on the function selector). The diode test apples a small current, i.e., a few milliAmperes, that flows from the Red to Black lead and the meter reads the junction voltage. What voltages to you read for VBC (1 pt) and VBE (1 pt)? Document this.

8-2 Emitter follower

Wire up an NPN transistor as an emitter follower (Figure 8.2).

Drive the follower with a sine wave of about 1 V amplitude and 1 kHz that is symmetrical about zero volts and look with a scope at the poor replica that comes out. Show a SCREENSHOT (1 pt). Explain exactly why this happens (1 pt).

If you turn up the waveform amplitude you will begin to see bumps below ground. Show a SCREENSHOT (1 pt). How do you explain these (2 pt)? (Hint: see VBE breakdown specification in the data sheet for the 2N3904 transistor).

Figure 8.2: Emitter follower. The small base resistor is often necessary to prevent oscillations.

Now try connecting the emitter return, i.e., the point marked VEE, to -15 V instead of ground, and look at the output. Document with a SCREENSHOT (1 pt) and explain the improvement (2 pt).

8-3 Input and output impedance of emitter follower

Measure Rin, the resistance looking into the base, and Rout, the resistance looking back into the emitter, for the follower in Figure 8.3.

Figure 8.3: Follower: circuit for measuring Rin and Rout

Using the circuit in Figure 8.2, replace the small base resistor with a 10 k resistor to simulate a signal source of moderately high impedance, i.e., low current capability (Figure 8.3).

Measure Rout, the output impedance of the emitter follower, as follows.  Connect a 1 k load to the output and observe the drop in output signal amplitude. It is best to use a small input signal, less than a volt. Document with a SCREENSHOT (1 pt) and report Rout (1 pt). Explain why you should use a blocking capacitor (1 pt).

Suggestions for measurement of Rout:

- If you view the emitter follower’s output as a signal source in series with RoutThevenin, the 1 k resistive load forms a voltage divider at signal frequencies, where the impedance of the blocking capacitor is negligibly small.

- The attenuations are likely to be small. To measure them we suggest that you AC couple the output signal to the scope, to ensure centering, then attach the load resistor and read the amplitude.

Remove the 1 k load. Now measure Rin, which is the impedance looking into the transistor’s base, by looking alternately at both sides of the 10 k input resistor. For this measurement the 3.3 k emitter resistor is also the “load” resistor Again, use a small signal. Document with a SCREENSHOT (1 pt). Does the result make sense (1 pt)?

When you have measured Rin and Rout, infer your transistor’s β (1 pt) (Hint: see textbook or class handout on emitter follower for help). Does the result make sense (1 pt)?

8-4 Transistor current gain

You saw how the transistor’s current gain, β, modified impedances in section 6.3. Now measure β directly at several values of IC with the circuit shown in Figure 8.4. Use the DMM as an Ammeter. The 4.7 kΩ and 1 kΩ resistors limit the currents. Which currents do they limit, and to what values (1 pt)?

Try various values for R, using either the resistor substitution box or individual resistors. Try 1 MΩ, 500 kΩ, 200 kΩ, 100 kΩ, 50 kΩ and 20 kΩ; note that when the resistor is too small the transistor will no longer be in the active state as IC is too large. Estimate the collector current IC for each different IB (3 pts; 1/2 for each resistor) Don’t bother to measure IB directly; just assume VBE = 0.6 V and calculate IB as (5.0V - 0.6V) / (R + 4.7k). Estimate β as an average across measurements; a plot of IC versus IB with a fitted line would be best (2 pts).

Figure 8.4: Circuit for measurement of β.

8-5 Common emitter amplifier

Figure 8.5: Common-emitter amplifier.

Wire up the common emitter amplifier shown in Figure 8.5 (Note: see design in lecture notes). What is the expected voltage gain (Vout/Vin) (1 pt)? Check it out using an input at ~ 1 kHz with an amplitude of a few hundred milliVolts. Is the signal’s phase inverted? Document your work with SCREENSHOTS emphasizing the AC part and the DC offsets (2 pts).

Is the collector quiescent operating point at the expected values (Note: see design in lecture notes) (1 pt)? What should the output impedance be (1 pt)? Check it by connecting a resistive load as in section 8.3 (Note: you will need to estimate a good value for the load) through a blocking capacitor. The capacitor lets you test impedance at signal frequencies without altering the biasing scheme. Describe your results (1 pt) and document with a SCREENSHOT(1 pt).

8.6 Push-pull amplifier

NPN and PNP transistors can be connected in series that, properly biased, can function as a linear amplifier over a range of voltages (Figure 8.6) . The tricky part is the crossing near zero volts where one transistor is turning off as the other has not yet turned on.

Build the push-pull amplifier circuit shown in Figure 8.7 Drive it first with a sine wave of 100 - 500 Hz (Vin). Use scope probes to simultaneously look at the output of the op-amp (point "A") and the output of the push-pull stage (Vout); make sure you have at least a few volts of output and that the function generator is set for zero DC offset. You should see classic cross-over distortion; highlight this on SCREENSHOTs with different output amplitudes (2 pts).

Figure 8.6: NPN and PNP transistors span positive and negative VCE, except for |VCE| < |VCE(sat)|.

Op-amp feedback only

Figure 8.7: Amplifier with push-pull buffer.

Listen to this waveform on a small speaker. But before you drive the speaker you should determine the maximum safe amplitude, given the following power ratings:

transistors: 350 mW

speaker: 250 mW

With system-wide feedback

Now reconnect the right side of the 100 k feedback resistor across the full amplifier, i.e., to Vout rather than the output the op-amp ("A" in Figure 9.2), and once again look at the push-pull output. The crossover distortion should be almost eliminated. What should the signal at the output of the op-amp look like? What does it look like? Document your observations with SCREENSHOTs. (1 pt).

Try listening with the speaker again. Better?

System-wide feedback and feed-through connectivity

The near complete elimination of crossover distortion makes use of a linear feed-through so that the op-amp can directly drive the load when the currents required are very small. Add a 100   feed-through resistor as component (Figure 8.8). Is the distortion gone? Document your observation with a SCREENSHOT (1 pt).

Combined circuits

Hook up the output of the modulation circuit that you built for exercise 6.2 - hopefully saved and still operative! - as the input to the push-pull amplifier with full feedback (Figure 8.8). Does the 10 k input resistance of the push-pull amplifier load the output of the modulator circuit? By how much? Document the output with a SCREENSHOT (2 pts).

Figure 8.8: Amplifier with system-wide feedback across push-pull buffer and feed-through resistor.

Fun

Try to make your combined device sound like a few bars from the late Keith Emerson, shown above in his musical prime next to his Moog synthesizer. Be careful, too high a gain and you will blow out the transistors and/or the speaker!

36 point total