Notes on Relaxation Oscillators

Physics 120, David Kleinfeld, Spring 2017

This is a basic oscillator circuit in which the voltage across a capacitor relaxes toward a time varying target voltage. Consider the circuit below, using a high gain op-amp as a comparator:

The time-varying voltage at V+ serves as a target level that is a fraction R2/(R1+R2) of the output voltage Vout.  The time-varying voltage at V- heads toward this value and when it exceeds the value, the output of the amplifier changes sign, thus the target changes sign, and the voltage at V- heads toward this new value.

We know that Vout=AV+-V- where A >> 1.

Since Vout is bounded by Vsupply. this means that

Vout= +Vsupply        V+>V-.-Vsupply         V-<V+.

The voltage divider assures that the value of  V-, which can asymptote at V- = Vout, will be compared to a value for V+ that is smaller than Vout, i.e.,  V+= R2R1+R2 Vout.

V-(t) will evolve in time according to:

CdV-dt+ V-- VoutR=0

with τ ≡RC. Thus:

dV-dt+ 1τV-= 1τVout

The solution to this homogeneous part of this equation is:

V-t=V-0e-tτ .

so that the full solution is:

V-t=V-0e-tτ+ ot 1τVoute-t-xτ dx .

We take t = 0 as the time of the last transition and consider the interval of time up to the next transition, i.e., t = T/2, so that Vout is a constant. The value of V_(0) just after the transition, i.e., V_(0+), is opposite in sign to that of Vout, so

V-(0)= -R2R1+R2 Vout.

Thus:

V_(t) = -R2R1+R2Vout e-tτ+Vout1-e-tτ

At t = T/2 the value of V_(t) will reach the threshold level

V-(T2)= +R2R1+R2 Vout

so

R2R1+R2 Vout = -R2R1+R2Vout e-T2τ+Vout1-e-T2τ

or

T=2 τ log1+2R2R1.

As a partial check,  when R2⟶∞  we have  T⟶∞.

Also,  when R2⟶0  we have  T⟶4 τR2R1.